Chapter 13: Home Network Basics

Professional Setup and Troubleshooting for Your Digital Life

"When assisting a client in this situation, there are several methods to retrieve Wi-Fi credentials and account login information from the existing computer." - From our professional network procedures

This line from our service manual tells an important story: home networking isn't just about connecting devices - it's about creating a reliable foundation for your entire digital life. Professional-level setup and troubleshooting can save you from countless connectivity headaches.

What You'll Learn in This Chapter:

- How to set up a new router like a professional

- Understanding WiFi networks and how they actually work

- Professional network migration techniques (when replacing equipment)

- IoT and smart home device connectivity (the modern challenge)

- Troubleshooting wireless problems systematically

- When DIY ends and professional network design begins

The Reality of Home Networking Today

Based on thousands of network setups and service calls over 20+ years

Why Home Networks Matter More Than Ever

Your network is now your lifeline:

- Work-from-home depends on reliable connectivity

- Smart home devices need constant internet access

- Streaming services require consistent bandwidth

- Family members expect WiFi "everywhere" in the house

Real Stories from the Field

The $2,000 Lesson: Client bought an expensive mesh system, tried to set it up themselves. Three weeks of frustration later, they called us. Problem: ISP-provided modem/router combo was conflicting with new mesh system, creating double-NAT issues. Professional setup took 2 hours and solved months of connectivity problems.

The Smart Home Nightmare: Customer had 15 smart devices that "worked fine" until they replaced their router. New router's default security settings blocked device-to-device communication. Professional configuration restored all devices in one service call. DIY attempts had lasted weeks.

The Work-From-Home Crisis: Client's video calls kept dropping during important meetings. Router was fine, internet speed was good, but router was in basement and work desk was upstairs. Professional solution: added wired access point in office area. Problem solved permanently.

Professional Perspective: What Makes Networks Actually Work

Most networking problems aren't about speed - they're about:

- Coverage - WiFi not reaching where you need it

- Compatibility - Old devices not working with new security settings

- Configuration - Settings that conflict with device requirements

- Interference - Other electronics disrupting wireless signals

Understanding Your Home Network: The Professional View

Fundamentals that most people never learn

Network Components: What Does What

Internet Service Provider (ISP) Equipment:

- Modem - Converts internet signal for your home (cable, fiber, DSL)

- Router - Creates your local network and WiFi

- Combo Unit - Many ISPs provide modem + router in one box

Your Network Equipment:

- Router - Manages all network traffic and creates WiFi

- Access Points - Extend WiFi coverage to additional areas

- Switches - Add more wired ports for devices

- Range Extenders - Boost WiFi signal (often problematic)

Professional Network Topology

Internet → Modem → Router → Your Devices

What actually happens:

- Internet comes to modem (cable, fiber, or phone line)

- Modem converts signal to ethernet

- Router creates your private network (192.168.1.x or similar)

- Router provides WiFi and wired connections

- Devices get local IP addresses and internet access

IP Addresses: Understanding Your Network's Language

Professional IP Address Ranges: From our network migration procedures

Common Home Network Ranges:

- 192.168.1.x (most common - Linksys, Netgear default)

- 192.168.0.x (common - D-Link, some Netgear)

- 10.0.0.x (Apple, some business routers)

- 10.1.10.x (some cable company equipment)

Why this matters: All devices on your network need unique addresses in the same range to communicate.

Professional Router Setup: Step-by-Step

Adapted from our service procedures for Linksys, TP-Link, Netgear, and other brands

Pre-Setup Planning

Before you start, gather this information:

- ISP connection type (usually automatic/DHCP)

- Current WiFi network name and password (if replacing router)

- List of devices that will connect

- Location considerations (central, elevated, away from interference)

Phase 1: Physical Setup

Professional Installation Approach:

Step 1: Optimal Router Placement

- Central location in home (not in basement or closet)

- Elevated position (shelf, desk, not on floor)

- Away from interference (microwaves, baby monitors, thick walls)

- Good ventilation (routers generate heat)

Step 2: Physical Connections

- Connect power adapter to router and plug into power

- Connect ethernet cable from modem to router's WAN/Internet port

- Wait 2-3 minutes for router to fully boot up

- Check status lights - should indicate power and internet connection

Professional Tip: If upgrading from ISP-provided equipment, you may need to call your ISP to register the new router's MAC address.

Phase 2: Initial Configuration

Professional Configuration Process:

Step 1: Access Router Interface

- Connect computer to router via WiFi or ethernet

  - Look for default network name on router label

  - Use default password on router label

- Open web browser and go to router's IP address

  - Common addresses: 192.168.1.1 or 192.168.0.1

  - Check router documentation for specific address

- Login with default credentials

  - Usually found on router label or in documentation

  - Common: admin/admin, admin/password, or admin/(blank)

Step 2: Run Setup Wizard Most modern routers have setup wizards that guide you through:

Internet Setup:

- Usually select "Dynamic IP (DHCP)" unless ISP provides specific settings

- Test internet connection before proceeding

WiFi Network Setup:

- Change network name (SSID) to something unique but not revealing

  - Good: "SmithFamily" or "HomeNetwork"

  - Bad: "NetgearRouter" or "BankManagerJones"

- Set strong WiFi password:

  - Minimum 12 characters

  - Mix of letters, numbers, and symbols

  - Use WPA2 or WPA3 security (never WEP or Open)

Step 3: Essential Security Settings

Change Default Admin Password:

- Find "Administration" or "Management" section

- Change default login password to something strong

- Write it down securely - you'll need it later

Update Firmware:

- Check for firmware updates in router settings

- Install any available updates (improves security and performance)

- Set automatic updates if available

Phase 3: Professional Optimization

Advanced Settings for Better Performance:

WiFi Channel Selection:

- 2.4GHz: Use channels 1, 6, or 11 (non-overlapping)

- 5GHz: Usually automatic selection works well

- Check for interference with WiFi analyzer app

Quality of Service (QoS):

- Enable QoS if you have bandwidth limitations

- Prioritize work devices over entertainment devices

- Set bandwidth limits for streaming devices if needed

Guest Network Setup:

- Enable guest network for visitors

- Different password from main network

- Isolate guest devices from main network

Network Migration: Professional Changeover Techniques

When replacing networking equipment without losing connectivity

The Professional Migration Process

Adapted from our enterprise network migration procedures

Phase 1: Pre-Migration Documentation

Critical Information to Record: From our professional migration checklist

Current Network Settings:

- Router login information (username/password)

- DHCP settings (IP range, gateway, DNS)

- Current WiFi SSID(s) and passwords

- Static IP assignments (printers, NAS, servers, cameras)

- Port forwarding rules (if any)

- VPN configurations (if any)

Device Inventory:

- List all connected devices and their current IP addresses

- Note devices requiring static IPs (printers, security cameras, NAS)

- Document any special configurations

Professional Command for WiFi Password Recovery: From our service procedures

Windows WiFi Password Retrieval:

- Open Command Prompt as Administrator

- Type: netsh wlan show profiles

- Note your network name (SSID)

- Type: netsh wlan show profile name="NETWORK_NAME" key=clear

- Look for "Key Content" - this is your WiFi password

Phase 2: New Equipment Pre-Configuration

Professional Pre-Setup Process: Configure new equipment before installation

Air-Gapped Setup:

- Set up new router separately (not connected to internet yet)

- Access router's setup interface

- Configure to match old network:

  - Same IP address range

  - Same WiFi SSID and password

  - Same DHCP settings

  - Same security settings

Why this works: Devices won't know network hardware changed - same names, passwords, and IP ranges.

Phase 3: Professional Cutover Process

Minimizing Downtime:

Step 1: Preparation

- Schedule cutover during low-usage time

- Notify family/users of planned outage

- Have old equipment documentation ready for reference

Step 2: Hardware Changeover

- Power off old equipment completely

- Install new equipment in same location

- Connect all cables (power, ethernet, etc.)

- Power on and wait for full startup

Step 3: Verification and Troubleshooting

- Test internet connectivity from router

- Verify WiFi devices reconnect automatically

- Check wired devices for connectivity

- Verify all services work (printers, streaming, etc.)

Professional Troubleshooting:

- Devices not connecting: Check if network names and passwords match exactly

- Some devices missing: May need to forget/reconnect WiFi on some devices

- Services not working: Check port forwarding and special configurations

Smart Home and IoT Device Connectivity

Professional solutions for modern networking challenges

Understanding IoT Connectivity Challenges

Based on our IoT troubleshooting experience and blog content

Why Smart Devices Are Different:

- Prefer 2.4GHz WiFi (longer range, better wall penetration)

- Require specific security settings (some don't work with WPA3)

- Need device-to-device communication (isolation breaks functionality)

- Sensitive to advanced router features (band steering, load balancing)

Professional IoT Setup Strategy

The Professional IoT-Friendly Configuration: Adapted from our wireless troubleshooting guide

Network Band Configuration:

- Create separate 2.4GHz network or disable band steering

- Set minimum bitrate to 6 Mbps (accommodates slower IoT devices)

- Use WPA2 security (not WPA3) for maximum compatibility

Router Settings for IoT Success:

- Disable client load balancing (prevents forced disconnections)

- Set 802.11w (Protected Management Frames) to "Optional" not "Required"

- Enable multicast/broadcast traffic (required for device discovery)

- Disable SSID isolation (prevents device-to-device communication)

DHCP and Network Services:

- Ensure DHCP is functioning correctly

- Allow access to required domains:

  - device-metrics-us.amazon.com

  - alexa.amazon.com

  - a0.awsstatic.com

- Set DNS to reliable service (Google: 8.8.8.8, Cloudflare: 1.1.1.1)

Temporary IoT Test Network

Professional Troubleshooting Setup: When devices won't connect, create this test environment

Simplified Network Configuration:

- 2.4GHz only

- WPA2 encryption (no WPA3)

- Band steering off

- 802.11w disabled

- Minimum bitrate at 6 Mbps

- No VLANs or client isolation

- Simple network name (no special characters)

Professional Insight: If devices connect to this simplified setup, gradually add security features back until you find the problematic setting.

Common Smart Device Setup Issues

Amazon Echo/Alexa Devices:

- Require 2.4GHz during initial setup

- Band steering can push to 5GHz and fail setup

- Need access to Amazon services (check firewall settings)

Smart Home Hubs (SmartThings, Hubitat):

- May need port forwarding for remote access

- Require multicast for local device discovery

- Static IP recommended for reliability

Security Cameras:

- Often require 2.4GHz

- Need consistent internet access for cloud features

- May require specific ports open

Wireless Troubleshooting: Professional Systematic Approach

Field-tested procedures for resolving connectivity issues

The Professional Troubleshooting Process

Phase 1: Information Gathering

- When did the problem start? (after change, gradually, suddenly)

- Which devices are affected? (all, some, specific types)

- What changed recently? (new router, ISP work, new devices)

- Is it location-specific? (whole house, specific rooms, distance-related)

Common Connectivity Problems and Professional Solutions

Problem: WiFi Connected But No Internet

Professional Diagnostic Approach:

Step 1: Verify Internet at Router

- Connect computer directly to router via ethernet

- Test internet access from wired connection

- If wired works: WiFi configuration issue

- If wired doesn't work: ISP or router issue

Step 2: Check DNS Settings

- Open Command Prompt

- Type: nslookup google.com

- If fails: DNS problem - change DNS servers

- Windows DNS change: Network settings → Change adapter options → Properties → TCP/IPv4 → Use these DNS servers: 8.8.8.8, 8.8.4.4

Step 3: IP Address Issues

- Check if device has valid IP: ipconfig (Windows) or Network settings

- If IP starts with 169.254: DHCP failure - restart router

- If no IP at all: WiFi connection issue

Problem: Slow WiFi Performance

Professional Performance Analysis:

Speed Testing Protocol:

- Test at different locations (near router, far from router)

- Test at different times (peak vs off-peak hours)

- Test wired vs wireless (ethernet vs WiFi comparison)

- Test different devices (isolate device-specific issues)

Common Performance Solutions:

- Channel interference: Change WiFi channel (use 1, 6, or 11 for 2.4GHz)

- Outdated device: Older devices limit network speed for all users

- Router placement: Move router to central, elevated location

- ISP throttling: Contact ISP if speeds consistently below plan

Problem: Intermittent Connectivity

Professional Intermittent Issue Diagnosis:

Environmental Factors:

- Time-based patterns: Interference from other devices (microwaves, baby monitors)

- Location patterns: Weak signal areas, construction interference

- Device patterns: Specific device types having issues

Router Health Check:

- Monitor router temperature (overheating causes drops)

- Check power adapter (failing adapters cause instability)

- Update firmware (fixes bugs causing intermittent issues)

- Check logs (router logs show disconnect patterns)

Advanced Troubleshooting Tools

Professional Network Analysis:

WiFi Analyzer Tools:

- WiFiInfoView (Windows) - shows all nearby networks and channels

- WiFi Explorer (Mac) - professional WiFi analysis

- WiFi Analyzer (Android) - mobile network scanning

Network Testing Commands:

- Ping test: ping google.com (tests basic connectivity)

- Traceroute: tracert google.com (shows network path)

- Network reset: netsh winsock reset (Windows network stack reset)

Professional Network Security

Protecting your digital life

Essential Security Settings

Router Security Checklist:

Access Control:

- Change default admin password (strong, unique password)

- Disable WPS (if not actively using - security vulnerability)

- Disable remote management (unless specifically needed)

- Enable automatic firmware updates

WiFi Security:

- Use WPA3 or WPA2 security (never WEP or Open)

- Strong WiFi password (12+ characters, mixed case, numbers, symbols)

- Hide SSID broadcast (optional - minimal security benefit)

- MAC address filtering (advanced - high maintenance)

Network Security:

- Enable firewall (usually enabled by default)

- Disable unnecessary services (UPnP, file sharing if unused)

- Guest network isolation (prevent guest access to main network)

- Regular security updates (firmware and connected device updates)

Professional Security Monitoring

Signs of Network Compromise:

- Unusual internet activity (slow speeds, unexpected data usage)

- Unknown devices on network (check router's device list)

- Changed passwords or settings you didn't modify

- Antivirus alerts about network activity

Monthly Security Maintenance:

- Review connected devices (remove unknown devices)

- Check for firmware updates (router and device updates)

- Review network logs (if comfortable reading them)

- Change WiFi password (quarterly for high-security environments)

Professional Network Testing Tools

Advanced diagnostics for serious network issues

Why Professional Network Testers Matter

Tools that separate professional diagnosis from guesswork

Professional network testing tools solve problems that software can't:

- Physical cable problems (breaks, shorts, incorrect wiring)

- Network performance issues (speed limitations, interference)

- Power over Ethernet (PoE) problems (insufficient power, polarity issues)

- Cable length and quality assessment (professional installation verification)

Pockethernet: Professional-Grade Network Testing

From our professional field toolkit

What is Pockethernet? The Pockethernet is a smartphone-controlled network cable tester that brings laboratory-grade testing capabilities to field technicians. We use this tool regularly for professional network diagnosis.

Professional Capabilities:

Wiremap Testing: Professional cable verification

- Tests all 8 wires in ethernet cable for proper connections

- Identifies cable type (straight-through, crossover, or faulty)

- Locates specific faults (opens, shorts, crossed pairs)

- Requires wiremap adapter on far end for complete testing

Professional Interpretation of Wiremap Results:

- Green: Proper straight-through or crossover cable detected

- Yellow: Cable open (broken wire)

- White: Short circuit (wires touching)

- Blue: PoE power supply detected

TDR (Time Domain Reflectometer) Testing: Professional cable length and fault location

- Measures exact cable length (±20m accuracy)

- Locates breaks and shorts to specific distance

- Tests cable quality and impedance characteristics

- Single-ended testing (only need access to one end)

Professional TDR Applications:

- Cable length verification for billing and documentation

- Fault location in walls or conduit runs

- Quality assessment of installed cables

- Troubleshooting intermittent connections

Link Testing: Active network performance testing

- Tests actual network speeds (10/100/1000 Mbps capability)

- Verifies link negotiation and auto-speed detection

- Checks wire pair polarity and signal quality

- Measures network delay skew (should be <56ns per 100m)

Power over Ethernet (PoE) Testing: Critical for modern network installations

- Detects PoE voltage presence and type

- Measures actual power delivery to device

- Tests PoE class compatibility (Class 8, 50V shown in your screenshots)

- Verifies power polarity on all pairs

Basic Pockethernet Usage Demonstration:

Step 1: Quick Cable Test (No App Required) Professional field technique for rapid testing

- Power on Pockethernet device

- Connect to cable or network port to test

- Press power button once to start quick test

- Observe LED results:

  - Cable/PoE LED: Green=good cable, Yellow=open, White=short, Blue=PoE detected

  - Link LED: Green=network link established

  - Network LED: Green=DHCP address obtained

Step 2: App-Based Testing (Detailed Analysis) Full professional diagnosis capabilities

- Install Pockethernet 2 app on smartphone

- Connect via Bluetooth to device

- Select test type:

  - Wiremap: Cable continuity and wiring verification

  - TDR: Length measurement and fault location

  - Link: Speed and performance testing

  - PoE: Power delivery analysis

- Run tests and save reports for documentation

Professional Use Cases:

- New cable installation verification (before finishing wall work)

- Troubleshooting existing network problems (intermittent connections)

- Performance optimization (identifying speed bottlenecks)

- PoE device troubleshooting (smart cameras, access points, phones)

Netool.io Network Testing Solutions

Modern cloud-connected network diagnostics

What is Netool.io? Netool.io devices are cloud-connected network testers that combine traditional cable testing with modern internet connectivity analysis and remote monitoring capabilities.

Professional Capabilities:

Comprehensive Network Testing:

- Cable testing and wiremap analysis (similar to Pockethernet)

- Internet connectivity testing (speed tests, latency measurement)

- Network path analysis (traceroute, DNS testing)

- Cloud reporting and monitoring (remote access to test results)

Advanced Features:

- WiFi network analysis (signal strength, channel usage, interference detection)

- Network security scanning (open ports, service discovery)

- Continuous monitoring (long-term network health tracking)

- Multi-location testing (compare performance across sites)

Basic Netool.io Usage:

Step 1: Device Setup

- Connect device to network (wired or wireless)

- Register device on netool.io cloud platform

- Configure testing parameters via web interface

- Set up automated testing schedules

Step 2: Cable and Physical Testing

- Connect to cable under test

- Run wiremap and continuity tests

- Perform length measurements

- Check PoE power delivery

Step 3: Network Performance Analysis

- Test internet speed and latency

- Analyze network path and routing

- Monitor long-term performance trends

- Generate professional reports

Professional Applications:

- Remote site monitoring (unattended testing)

- Customer network validation (proof of service quality)

- Managed service monitoring (ongoing network health)

- Troubleshooting support (remote diagnosis capability)

Klein VDV Scout Pro Series

Professional-grade cable verification tools

Klein VDV Scout Pro 3: Referenced in our professional toolkit

- Advanced cable testing for data and video cables

- Tone generation and tracing for cable identification

- Built-in digital multimeter for voltage and continuity testing

- Rugged construction for field use

Professional Applications:

- Commercial cable installation verification

- Structured cabling system testing

- Cable tracing in complex installations

- Multi-cable testing with remote units

Professional Testing Workflow

How professionals approach network diagnosis

Phase 1: Physical Layer Testing

Using professional cable testers:

- Verify cable integrity with wiremap testing

- Measure cable lengths and verify installation specs

- Test PoE power delivery for powered devices

- Document all physical layer results

Phase 2: Network Layer Testing

Using network analysis tools:

- Verify IP addressing and DHCP functionality

- Test network speeds and performance

- Check internet connectivity and DNS resolution

- Analyze network path and identify bottlenecks

Phase 3: Performance Optimization

Based on testing results:

- Replace faulty cables identified by testing

- Optimize network configuration based on performance data

- Relocate equipment based on signal strength measurements

- Document improvements and verify with retesting

When Professional Testing Tools Are Essential

Professional Installation Verification:

- New cable runs need professional verification before closing walls

- PoE device installations require power delivery verification

- High-performance networks need speed and quality verification

- Commercial installations require professional documentation

Complex Troubleshooting:

- Intermittent network problems that basic tools can't identify

- Performance issues affecting business operations

- Multi-building networks with long cable runs

- Mixed technology environments (copper, fiber, wireless)

Professional Service Applications:

- Customer proof of service quality and installation

- Warranty verification of cable and equipment performance

- Regulatory compliance documentation

- Ongoing monitoring and maintenance contracts

Professional Testing Investment

Cost-Benefit Analysis:

- Pockethernet: ~$200-400 (excellent for small business/advanced home users)

- Netool.io devices: ~$300-600 (best for ongoing monitoring needs)

- Klein VDV Scout Pro: ~$400-800 (commercial installation verification)

When the investment makes sense:

- Regular network troubleshooting needs

- Multiple property management

- Small business IT support

- Professional installation verification

- Home network enthusiasts with complex setups

Professional Service Alternative: If you don't need these tools regularly, professional network diagnosis typically costs $150-300 per visit, which often includes testing with professional-grade equipment.

When to Call a Professional

Recognizing the limits of DIY networking

Professional Network Design Scenarios

Call a Professional When:

Complex Requirements:

- Multiple buildings or large homes requiring coverage

- Business networking needs (VLANs, managed switches, enterprise WiFi)

- High-density device environments (20+ smart home devices)

- Professional work requirements (video conferencing, VPN, server hosting)

Technical Challenges:

- ISP equipment integration problems (double-NAT, compatibility issues)

- Persistent connectivity issues after trying standard troubleshooting

- Network performance problems affecting productivity

- Security concerns (suspected intrusion, malware on network)

Infrastructure Projects:

- Wired network installation (ethernet cables through walls)

- Outdoor WiFi coverage (proper weatherproofing and power)

- Network rack setup (professional cable management, enterprise equipment)

- Mesh system optimization (professional site survey and configuration)

Professional Service Selection

Questions to Ask Network Professionals:

- "Do you perform site surveys?" (Professional assessment of coverage needs)

- "What brands do you recommend and support?" (Ongoing support relationship)

- "Do you provide network documentation?" (Passwords, configurations, diagrams)

- "What's your approach to security?" (Professional security practices)

- "Do you offer ongoing support?" (Maintenance and troubleshooting services)

Professional Network Services Include:

- Site survey and design (coverage analysis and equipment placement)

- Professional installation (cable running, equipment mounting, configuration)

- Performance optimization (channel analysis, interference mitigation)

- Security configuration (enterprise-grade security, VPN setup)

- Documentation and training (network diagrams, password management, user training)

Chapter Summary: Your Network Foundation

Critical Concepts for Home Networking:

Network Fundamentals:

- Router creates your private network with local IP addresses (192.168.x.x)

- Modem connects to internet, router manages your local devices

- WiFi is just wireless access to your wired network

- All devices need unique IP addresses in same range to communicate

Professional Setup Principles:

- Location matters more than equipment - central, elevated, unobstructed

- Configuration compatibility - match settings to device requirements

- Security by design - strong passwords, current firmware, disabled unnecessary features

- Documentation is essential - record settings, passwords, and configurations

Your Professional Networking Toolkit:

Essential Knowledge:

- How to access router settings (IP address, login credentials)

- WiFi password recovery techniques for Windows

- Basic IP addressing concepts and troubleshooting

- Professional channel selection (1, 6, 11 for 2.4GHz)

Troubleshooting Process:

- Document the problem systematically

- Test at the router level first (wired connection)

- Isolate variables (location, device, time, settings)

- Use professional diagnostic tools (ping, WiFi analyzers)

- Know when to escalate to professional service

Professional Insights:

"Home networking problems are rarely about speed - they're about compatibility, coverage, and configuration. A $50 router properly configured and positioned will outperform a $300 router thrown in a closet with default settings."

"The rise of smart home devices has changed everything about home networking. What worked for laptops and phones doesn't work for IoT devices. Professional-level configuration is now required for basic home networks."

"Network security isn't about paranoia - it's about building good habits. Strong passwords, regular updates, and understanding your network layout will prevent 95% of home network security issues."

What's Next?

In Chapter 14, we'll dive into internet troubleshooting - when your network is fine but your internet connection isn't. You'll learn to distinguish between local network problems and ISP issues, and how to effectively communicate with technical support when professional help is needed.

Remember: A well-configured home network is invisible when it's working properly. If you're constantly thinking about your network, it's not configured professionally. Spend time setting it up right once, and enjoy years of reliable connectivity.

Quick Reference: Home Networking Essentials

Router Setup Checklist:

- Central, elevated location with good ventilation

- Strong WiFi password (WPA2/WPA3 security)

- Changed default admin password

- Firmware updated to latest version

- Guest network enabled and isolated

Common IP Address Ranges:

- 192.168.1.x (Linksys, Netgear default)

- 192.168.0.x (D-Link, some Netgear)

- 10.0.0.x (Apple, business equipment)

IoT-Friendly Settings:

- 2.4GHz network available (separate or band steering off)

- WPA2 security (WPA3 for newer devices only)

- 802.11w set to "Optional" not "Required"

- Client isolation disabled

- Multicast traffic enabled

Professional Help When:

- Multiple building coverage needed

- Persistent connectivity issues after troubleshooting

- Business requirements (VLANs, enterprise security)

- Network performance affects productivity

WiFi Password Recovery (Windows):

netsh wlan show profiles

netsh wlan show profile name="NETWORK_NAME" key=clear

Chapter word count: ~4,900 words