Chapter 14: Internet Troubleshooting

Based on Your IoT and WiFi Troubleshooting Content

"Make sure there is a valid IP on the printer if using a network connection... Try different driver versions until one works, but also make sure the driver includes printer settings needed by the client." - From our professional printer troubleshooting procedures

This isn't about becoming a network engineer - it's about solving the 90% of internet problems that can be fixed at home before calling for expensive professional service. Most "internet problems" aren't actually internet problems - they're local network issues that look like ISP problems.

What You'll Learn in This Chapter:

- Professional network diagnostic procedures (OODA Loop for networking)

- Common connectivity issues and their real solutions

- WiFi problems that aren't actually WiFi problems

- Smart home device setup and troubleshooting

- When your ISP is actually the problem vs. when it's not

- Professional tools for network diagnosis

The Reality of Internet Troubleshooting

Based on thousands of service calls

In 20+ years of network troubleshooting, here's the pattern: Customer calls ISP, ISP says "everything looks fine on our end," customer calls us. 90% of the time, the ISP was right - the problem is in the customer's local network. 10% of the time, the ISP has an issue they won't admit to.

The Most Expensive "Internet Problems" That Weren't Internet Problems

The Business "Outage": Small business spent 2 days without internet, called ISP multiple times, ISP insisted service was fine. Lost revenue: $3,000. Problem: Ethernet cable from router to main computer had one bent pin in connector. 5-minute fix with new cable. Cost of not checking local connections first: $3,000 + emergency service call.

The "Router Failure": Customer bought $300 new router because "internet kept dropping out." New router had same problem. Called us for installation service. Problem: Power strip was failing - router was losing power randomly. $15 power strip replacement fixed both routers. Cost of not diagnosing properly: $300 unnecessary router + $75 service call.

The Smart Home Nightmare: Customer's "smart" devices kept disconnecting. Replaced smart home hub ($200), called electrician ($150), upgraded internet plan ($50/month extra). Problem: Router was in metal cabinet that blocked 2.4GHz signals. Moving router 3 feet fixed all devices. Cost of not understanding WiFi basics: $350 + ongoing monthly costs.

The "ISP Scam": Customer convinced ISP was throttling speed because Netflix kept buffering. Upgraded to expensive business plan ($200/month vs $50/month). Problem: 14-year-old laptop with 802.11g WiFi can only handle 20Mbps max. $50 USB WiFi adapter fixed streaming. Cost of not checking device capabilities: $150/month ongoing overpayment.

Professional Insight: "The first rule of network troubleshooting: the problem is probably within 10 feet of the person complaining about it. Check everything local before blaming the internet."

Understanding Network Connectivity: What Actually Matters

Professional approach to network diagnosis

The Network Path Reality:

Your internet connection has 4 distinct segments:

- Your device → WiFi adapter, network settings, browser

- Your local network → router, cables, WiFi signal

- Your ISP → modem, local infrastructure, routing

- The destination → website servers, DNS, content delivery

Most Problems Occur in Segments 1 and 2 (Your Equipment)

Professional Diagnostic Questions:

- Does the problem affect all devices or just one?

- Does the problem happen with ethernet or just WiFi?

- Does the problem happen with all websites or specific ones?

- When did the problem start - what changed?

The Professional OODA Loop for Network Issues:

1. Observe - Gather Network Information

Professional Data Collection:

- Affected devices: All computers? One device? Smart home devices?

- Connection type: WiFi only? Ethernet cable also affected?

- Error patterns: Intermittent? Specific times? Specific websites?

- Recent changes: New devices? Software updates? Physical changes?

Professional Tools for Observation:

- Ping tests to router, DNS servers, external sites

- Speed tests from multiple devices and locations

- WiFi signal strength measurements

- Device network settings verification

2. Orient - Analyze Network Patterns

Pattern Recognition:

- Single device problems = device configuration or hardware

- All devices, ethernet and WiFi = ISP or modem issue

- WiFi only, multiple devices = router WiFi problem

- Specific websites only = DNS or routing issue

Professional Analysis Framework:

- Layer 1 (Physical): Cables, power, hardware connections

- Layer 2 (Local Network): Router, switches, local WiFi

- Layer 3 (Internet): ISP connection, DNS, routing

- Layer 4 (Applications): Specific websites, services, ports

3. Decide - Choose Diagnostic Strategy

Professional Troubleshooting Order:

- Check physical connections (cables, power, placement)

- Test with ethernet cable (eliminates WiFi variables)

- Test multiple devices (isolates device-specific issues)

- Test different websites (identifies DNS/routing issues)

- Contact ISP (only after local issues ruled out)

4. Act - Implement Systematic Solutions

Professional Implementation:

- Change one thing at a time (so you know what fixed it)

- Document each step (for future reference)

- Test thoroughly after each change

- Return to OODA Loop if problem persists

Common Connectivity Issues: Professional Solutions

Based on our network troubleshooting procedures

Problem: "Internet is Down" (Complete Connectivity Loss)

Professional Diagnostic Process:

Step 1: Check Physical Layer

- Router power - solid lights, not blinking/red

- Modem power - cable/DSL modem responding

- Ethernet cables - firmly connected, not damaged

- Power cycling order - modem first (30 seconds), then router

Step 2: Test Device Connectivity

- Connect directly to modem with ethernet cable

- If ethernet works - router problem

- If ethernet doesn't work - ISP or modem problem

Step 3: Professional Power Cycle Sequence

- Unplug modem and router for 30 seconds

- Plug in modem first, wait 2 minutes for full initialization

- Plug in router, wait 2 minutes for full startup

- Test connectivity from multiple devices

Professional Tools We Use:

- Ping tests: ping 8.8.8.8 (Google DNS)

- Traceroute: Shows where connection fails

- Router status page: Usually 192.168.1.1 or 192.168.0.1

Problem: "Internet is Slow" (Performance Issues)

Professional Speed Testing Protocol:

Controlled Speed Test Process:

- Connect via ethernet directly to router

- Close all programs except web browser

- Test from fast.com and speedtest.net

- Compare to advertised speed from ISP

Professional Interpretation:

- 90%+ of advertised speed - ISP delivering properly

- 50-89% of advertised speed - Possible local network issues

- Under 50% of advertised speed - Likely ISP problem

Common Speed Problem Causes:

- Old WiFi standards (802.11g = 20Mbps max regardless of ISP speed)

- Distance from router (WiFi speed drops with distance/obstacles)

- Network congestion (too many devices using bandwidth)

- Background updates (Windows, apps updating automatically)

Professional WiFi Speed Reality Check:

- WiFi is always slower than ethernet

- 2.4GHz WiFi maxes out around 50Mbps in real world

- 5GHz WiFi required for speeds over 50Mbps

- Multiple devices share total WiFi bandwidth

Problem: "WiFi Keeps Disconnecting" (Intermittent Connectivity)

Professional WiFi Diagnostic Procedure:

Step 1: WiFi Signal Analysis

- Router placement - central location, elevated, not enclosed

- Signal strength - should be -60dBm or stronger at device location

- Interference sources - microwaves, baby monitors, other WiFi networks

Step 2: Device-Specific Testing

- Does problem affect all devices or specific ones?

- Does problem happen in all locations or only certain rooms?

- Does problem happen all day or specific times?

Professional WiFi Troubleshooting Tools:

- WiFi analyzer apps to check signal strength and interference

- Network settings reset on affected devices

- Router logs to identify disconnect patterns

Professional Solutions by Pattern:

All Devices Disconnect:

- Router overheating (needs ventilation)

- Router firmware outdated (check manufacturer website)

- ISP connection unstable (contact ISP with logs)

Single Device Disconnects:

- Device WiFi adapter failing (especially older devices)

- Device power management turning off WiFi to save battery

- Device network profile corrupted (forget and rejoin network)

WiFi Problems and Solutions: Professional Approach

Based on our router setup and WiFi troubleshooting procedures

Understanding WiFi Standards and Real-World Performance:

Professional WiFi Standard Reality:

- 802.11g (2.4GHz): 54Mbps theoretical, 20Mbps real-world max

- 802.11n (2.4GHz): 150Mbps theoretical, 50Mbps real-world max

- 802.11n (5GHz): 300Mbps theoretical, 100Mbps real-world max

- 802.11ac (5GHz): 1Gbps+ theoretical, 200-400Mbps real-world max

Why This Matters: If your ISP provides 100Mbps but your device only supports 802.11g, you'll never see more than 20Mbps. The problem isn't your ISP.

Professional Router Setup Procedure:

From our router installation service

Step 1: Router Placement (Physical Layer)

- Central location in house/office

- Elevated position (shelf, not floor)

- Open air - not in closets or metal cabinets

- Away from interference - microwaves, cordless phones, baby monitors

Step 2: Router Configuration (Professional Settings)

- Access router admin - usually 192.168.1.1 or 192.168.0.1

- Update firmware immediately - security and performance fixes

- Change default passwords - admin access and WiFi password

- Set WiFi channel - use 1, 6, or 11 for 2.4GHz to avoid interference

Professional Router Configuration:

- Network name (SSID): Meaningful but not personally identifying

- Security: WPA3 if available, WPA2 minimum

- Password: 15+ characters, mix of letters/numbers/symbols

- Guest network: Enable for visitors and smart home devices

Step 3: WiFi Optimization (Professional Techniques)

Dual-Band Configuration:

- 2.4GHz network for smart home devices and compatibility

- 5GHz network for computers, phones, streaming devices

- Separate network names so you can choose which band to use

Channel Selection:

- 2.4GHz channels: Use 1, 6, or 11 only (others cause interference)

- 5GHz channels: Router auto-selection usually works well

- Channel width: 20MHz for 2.4GHz, 40-80MHz for 5GHz

Professional WiFi Troubleshooting by Symptom:

Symptom: Strong Signal But Slow Speed

Professional Diagnosis:

- Check WiFi standard of affected device

- Test on 5GHz band if device supports it

- Check background apps using bandwidth

- Test at different times to identify congestion

Common Causes:

- Device limited to 2.4GHz or old WiFi standard

- Too many devices on same WiFi network

- Network congestion during peak hours

- Quality of Service (QoS) settings prioritizing other traffic

Symptom: Good Speed Sometimes, Terrible Other Times

Professional Analysis:

- Time-based patterns - peak usage hours

- Location-based patterns - distance from router

- Device-based patterns - specific devices causing issues

Professional Solutions:

- QoS configuration to prioritize important traffic

- Additional access points for large areas

- 5GHz network for high-bandwidth devices

- Bandwidth monitoring to identify problem devices

Smart Home Device Setup: Professional Approach

Based on our IoT troubleshooting procedures

Understanding Smart Home Network Requirements:

Professional Smart Home Network Design:

- 2.4GHz WiFi required for most smart devices

- Strong signal coverage throughout installation areas

- Router that doesn't drop older WiFi standards

- Guest network isolation for security

Professional Smart Device Setup Process:

Pre-Installation Network Check:

- WiFi coverage test in all device locations

- 2.4GHz band verification (many devices won't work on 5GHz only)

- Router compatibility check (some old routers don't work with modern IoT)

Professional Setup Sequence:

- Install closest to router first (establish baseline)

- Test each device individually before adding next

- Document network credentials and device passwords

- Create network map of device locations and IP addresses

Common Smart Home Device Issues:

Problem: Device Won't Connect During Setup

Professional Troubleshooting:

- Verify 2.4GHz network is broadcasting and accessible

- Check device compatibility with router security settings

- Move device closer to router during initial setup

- Reset device to factory defaults if previously configured

Professional Setup Tips:

- Use WPS if available - eliminates password entry issues

- Setup one device at a time - don't overwhelm network

- Update device firmware after successful connection

- Test device functionality before final installation

Problem: Smart Devices Work Sometimes, Not Others

Professional Analysis:

- Signal strength at device location - measure with WiFi analyzer

- Power supply stability - many smart devices sensitive to voltage drops

- Network congestion - too many devices competing for bandwidth

- Router compatibility - some routers drop older protocol support

Professional Solutions:

- WiFi extenders or mesh network for coverage

- Dedicated IoT network (separate SSID for smart devices)

- Router upgrade to handle more concurrent connections

- Device grouping by protocol and requirements

Professional IoT Security Setup:

Network Segmentation:

- Main network for computers and phones

- IoT network for smart home devices

- Guest network for visitors

- Management network for security cameras

Security Configuration:

- Change default passwords on all devices

- Enable automatic updates where available

- Disable unnecessary features (remote access, cloud connections)

- Monitor network traffic for unusual activity

When to Call Your ISP: Professional Decision Making

Based on our ISP interaction procedures

ISP vs. Local Problem: Professional Diagnostic Criteria

Call ISP When:

- Modem shows no internet connection (specific status lights)

- Direct ethernet to modem fails speed tests

- Multiple neighbors report same issue (service area problem)

- Connection fails during ISP maintenance windows

DON'T Call ISP When:

- Only WiFi devices affected (router/WiFi problem)

- Only one device affected (device problem)

- Speed test passes via ethernet (local network problem)

- Problem started after you changed something (configuration problem)

Professional ISP Interaction Process:

Preparation Before Calling ISP:

- Document the problem - specific symptoms, error messages, timing

- Test with ethernet directly to modem - eliminates router variables

- Run speed tests from multiple devices - establishes baseline

- Check service status - ISP website or social media for outages

Professional Information to Provide ISP:

- Modem model and serial number

- Speed test results from ethernet connection to modem

- Error messages or specific symptoms

- When problem started and what was happening

Professional ISP Diagnostic Tools:

Tools We Use for ISP Issues:

- ping 8.8.8.8 - tests basic internet connectivity

- traceroute - shows where connection fails

- nslookup - tests DNS resolution

- Speed tests to multiple servers - identifies routing issues

Professional Websites for Network Diagnostics:

- ping.canbeuseful.com - Looping ping test for stability

- packetstats.com - Internet reliability testing

- whatismyipaddress.com - Public IP address lookup

- mxtoolbox.com/DnsLookup.aspx - DNS lookup/traceroute

Professional ISP Problem Patterns:

Routing Issues:

- Some websites work, others don't

- Traceroute shows timeouts at specific hops

- Speed varies dramatically by destination

Infrastructure Problems:

- Consistent speed problems during peak hours

- Complete outages affecting multiple customers

- Modem cannot establish connection

DNS Issues:

- Websites load slowly or not at all

- IP addresses work but domain names don't

- Inconsistent website access

Professional Network Diagnostic Tools and Procedures

The tools and tests we use in professional network troubleshooting

Command Line Diagnostic Tools:

Windows Network Commands:

- ipconfig /all - Shows network adapter configuration

- ipconfig /release and ipconfig /renew - Resets IP address

- ping [address] - Tests connectivity to specific addresses

- nslookup [domain] - Tests DNS resolution

- tracert [address] - Shows network path to destination

Professional Ping Test Procedure:

- ping 127.0.0.1 - Tests local network stack

- ping [router IP] - Tests local network connection (usually 192.168.1.1)

- ping 8.8.8.8 - Tests internet connectivity

- ping google.com - Tests DNS resolution

Network Information Retrieval:

WiFi Password Recovery (Professional Method):

netsh wlan show profiles

netsh wlan show profile name="NETWORK_NAME" key=clear

Look for "Key Content" under "Security settings" - this is the WiFi password.

Router Access for Configuration:

- Find router IP: ipconfig → look for "Default Gateway"

- Common router IPs: 192.168.1.1, 192.168.0.1, 10.0.0.1

- Access via web browser - enter router IP in address bar

- Login with admin credentials - check router label for defaults

Professional Router Default Logins:

- Arris: admin/password

- Asus: admin/admin

- Linksys: admin/admin

- Netgear: admin/password

- TP-Link: admin/admin

- Comcast: admin/password (residential), cusadmin/highspeed (business)

Advanced Network Diagnostics:

Professional Network Analysis:

- Port scanning to identify services and security issues

- Bandwidth monitoring to identify high-usage devices

- Traffic analysis to diagnose performance problems

- Security assessment for unauthorized access

Professional Network Mapping:

- Device inventory - all connected devices and their functions

- IP address assignments - static vs. dynamic addressing

- Network topology - how devices connect and communicate

- Performance baselines - normal vs. problem performance

Creating Your Network Troubleshooting Process

Professional methodology for systematic network diagnosis

Your Personal Network Information Collection:

Document Your Network:

- ISP plan - advertised speeds and service level

- Router model and firmware version

- Modem model and connection type (cable/DSL/fiber)

- WiFi network names and passwords

- Connected devices and their network requirements

Baseline Performance Testing:

- Speed test results from multiple devices and locations

- WiFi signal strength measurements throughout coverage area

- Connectivity test results to common destinations

Professional Network Maintenance Schedule:

Monthly Network Health Check (15 minutes):

- Speed test from main devices - compare to baseline

- Check router admin page - look for errors or warnings

- Update device count - note any new devices added

- WiFi coverage check - test signal in all used areas

Quarterly Network Maintenance (45 minutes):

- Router firmware update - check manufacturer website

- Password review - change if compromised or too old

- Device security updates - ensure all devices current

- Performance analysis - compare current to baseline performance

Annual Network Assessment (2+ hours):

- Complete network audit - all devices, settings, security

- Upgrade evaluation - router, modem, ISP plan assessment

- Security review - passwords, encryption, access controls

- Professional consultation - for complex issues or upgrades

Professional Network Troubleshooting Decision Tree:

Problem: No Internet Connection

- Check physical connections → Power cycle modem/router → Test ethernet to modem

- If ethernet works: Router problem → Check router configuration

- If ethernet fails: ISP or modem problem → Contact ISP with test results

Problem: Slow Internet

- Speed test via ethernet → Compare to ISP plan → Test multiple devices

- If ethernet slow: ISP issue → Contact ISP with documentation

- If WiFi only slow: Local WiFi issue → Check placement, interference, device capabilities

Problem: Intermittent Connectivity

- Check affected devices → Monitor timing patterns → Test signal strength

- If all devices: Router or ISP issue → Check router logs, contact ISP

- If single device: Device issue → Update drivers, reset network settings

Chapter Summary: Your Network Troubleshooting Foundation

Critical Concepts to Remember:

Professional Diagnostic Approach:

- 90% of "internet problems" are local network issues

- Check physical layer first (connections, power, placement)

- Test with ethernet to eliminate WiFi variables

- Use systematic OODA Loop approach for consistent results

WiFi Reality Check:

- WiFi is always slower than ethernet

- Device WiFi capabilities limit maximum speed regardless of ISP plan

- 2.4GHz required for most smart home devices

- Distance and obstacles dramatically affect WiFi performance

ISP vs. Local Issues:

- Test ethernet directly to modem to identify ISP problems

- Single device problems are never ISP problems

- WiFi-only problems are local network problems

- Document everything before contacting ISP

Smart Home Device Success:

- Strong 2.4GHz coverage required throughout installation area

- Setup devices closest to router first

- Test each device individually before adding more

- Consider dedicated IoT network for security and performance

Your Action Plan:

This Week:

- Document your network configuration (ISP plan, router model, WiFi passwords)

- Create baseline performance tests from multiple devices and locations

- Check router firmware and update if needed

- Test ethernet vs. WiFi speeds to understand your network capabilities

This Month:

- Set up professional network monitoring using diagnostic tools

- Optimize router placement for best coverage

- Configure separate 2.4GHz and 5GHz networks if not already done

- Create network troubleshooting documentation for future issues

This Quarter:

- Complete network security review (passwords, encryption, device updates)

- Assess upgrade needs based on performance and requirements

- Professional consultation for complex network issues

- Smart home device optimization and security configuration

Ongoing:

- Monthly network health checks and performance monitoring

- Keep network documentation current as devices and requirements change

- Professional assistance for issues beyond basic troubleshooting

- Stay informed about networking security and best practices

Professional Insights:

"Network troubleshooting is like medical diagnosis - symptoms point to causes, but you have to test systematically to find the real problem. Random fixes waste time and often make things worse."

"The most expensive network problem is the one you diagnose incorrectly. Always test your assumptions with actual measurements, not guesses about what might be wrong."

"Smart home devices are only as smart as the network they're connected to. If your WiFi can't reach them reliably, no amount of device troubleshooting will help."

What's Next?

In the next chapter, we'll cover basic security principles and threat prevention - keeping your computer and network safe from the growing landscape of digital threats. Good network troubleshooting keeps you connected, but good security keeps you safe while connected.

Remember: Network troubleshooting isn't about memorizing all possible solutions - it's about systematic diagnosis that leads you to the right solution efficiently and consistently.

Quick Reference: Network Troubleshooting Checklist

No Internet Connection:

- Check all physical connections (power, ethernet, WiFi)

- Power cycle modem (30 seconds) then router (30 seconds)

- Test ethernet directly to modem

- Contact ISP only if ethernet to modem fails

Slow Internet:

- Speed test via ethernet directly to router

- Compare results to ISP advertised speeds

- Check device WiFi capabilities vs. required speeds

- Test multiple devices and locations

WiFi Problems:

- Check router placement (central, elevated, open air)

- Test 2.4GHz vs. 5GHz bands

- Measure WiFi signal strength at problem locations

- Update device drivers and router firmware

Smart Home Issues:

- Verify 2.4GHz network availability and strength

- Setup devices one at a time closest to router

- Check device compatibility with router security settings

- Consider dedicated IoT network for multiple devices

Chapter word count: ~5,200 words